# Mathematical Simulation of Nonlinear Oscillations

# of Viscoelastic Pipelines Conveying Fluid

# B.A. Khudayarov, F.J. Turaev2

1,2Department of  Higher Mathematics,”Tashkent institute of irrigation and agricultural mechanization engineers” NRU, Tashkent, 100000, Uzbekistan

Abstract. A mathematical model of the problem of nonlinear oscillations of a viscoelastic pipeline conveying fluid is developed in the paper. The Boltzmann-Volterra integral model with weakly singular kernels of heredity is used to describe the processes of pipeline strain. Using the Bubnov-Galerkin method, the mathematical model of the problem is reduced to the study of a system of ordinary integro-differential equations, where time is an independent variable. The solution of integro-differential equations is determined by a numerical method based on the elimination of the singularity in the relaxation kernel of the integral operator. Using the numerical method for unknowns, a system of algebraic equations is obtained. To solve a system of algebraic equations, the Gauss method is used. A computational algorithm is developed to solve the problems of the dynamics of viscoelastic pipelines with a flowing fluid. The algorithm of the proposed method makes it possible to investigate in detail the effect of rheological parameters on the character of vibrational strength of viscoelastic pipelines with a fluid flow, in particular, in the study of free oscillations of pipelines based on the theory of ideally elastic shells. On the basis of the computational algorithm developed, a set of applied computer programs has been created, which makes it possible to carry out numerical studies of pipeline oscillations. The influence of singularity in the heredity kernels and the geometric parameters of the pipeline on the vibrations of structures possessing viscoelastic properties is numerically investigated. It is shown that an account of viscoelastic properties of pipeline material leads decrease in the amplitude and frequency of oscillation. It is established that to reveal the influence of viscoelastic properties of structure material on the pipeline oscillations, it is necessary to use the Abel-type weakly singular kernels of heredity. The obtained results of numerical simulation can be used in the enterprises of oil and gas industries, as well as in design organizations.

Keywords: mathematical model, simulation, viscous-elasticity, integro-differential equations, numerical algorithm, pipeline, fluid, cylindrical shell, oscillatory process, relaxation kernel.

- Introduction

Currently, the oil and gas industry is often faced with the problems of repair, reconstruction and restoration of pipelines due to the impact of various external factors. One of the ways to solve this problem is the use of modern, resource-saving, environmentally friendly technologies, which include the use of non-metallic, in particular, polymer composite materials [1,2].

At present, the problem of considering the hereditary effects of deformable materials is of great theoretical and applied interest. Its solution is an effective application of the theory of viscoelasticity to real processes. Therefore, the methods and problems of the theory of hereditary elasticity attract great attention of researchers. There is a significant number of publications devoted to solving the problems of calculating the characteristics of viscoelastic thin-walled structures 3-21.

Mathematical simulation of dynamic problems of oscillations and the stability of viscoelastic systems is also a very relevant problem due to the fact that, on the one hand, the possibilities of using materials with pronounced viscoelastic properties in oil and gas industry and other branches of engineering are expanding, and, on the other hand, when using hereditary models [3] for describing the internal damping of material, the oscillation equations of elastic systems are written in the same form as for viscoelastic systems. Often when considering elastic systems, the internal friction of material is taken into account by the Voigt model, although it is known that even in systems with a finite number of degrees of freedom greater than unity, it leads to incorrect results, since for most materials the internal friction is virtually independent or, at least, weakly depends on the velocity of oscillations in a sufficiently wide frequency range. In this sense, a model that reflects hereditary properties [4] is more preferable.

Analysis of the stability of viscoelastic systems under dynamic effects causes great difficulties, since it is connected to integral-differential equations. In this case, when the relaxation value of material is small in comparison with unity, the averaging method is used [5] to solve the dynamic stability problem of a hinged supported rod. V.D. Potapov [4] in the study of the stability of integral-differential equations has used the Lyapunov exponent method.

Urbanowicz et al. [6] have studied the vibration behavior of viscoelastic pipelines. As a model of viscoelastic material, the Kelvin-Voigt generalized models are used. The results of numerical study of the processes in viscoelastic pipelines are presented. A comparison of numerical simulation and experimental data from known studiesis given.

…………………………………………………………………………………………………………………………………………………………

The wide use of new composite materials in nuclear power facilities, in aircraft construction, in oil and gas industry, in chemical production facilities and in other engineering industries requires further improvement of mechanical models of deformable bodies and the development of the methods for their calculation, taking into account the viscoelastic properties of thin-walled structures.

Thus, undoubted scientific and practical interest causes the construction of mathematical models that allow one to study the dynamic processes of viscoelastic pipelines conveying gas and fluid.

The aim of this study is to create a mathematical model, a numerical algorithm and a computer program for solving the problem of nonlinear oscillations of viscoelastic thin-walled pipelines of large diameter on the basis of shell theory [64, 65].

2. Mathematical model

Consider the behavior of a thin circular viscoelastic cylindrical shell conveying an ideal fluid with constant velocity U. The ordinary Kirchhoff-Love hypotheses are used and the deflections are assumed small in comparison with thickness (Fig. 1).

The Boltzmann-Volterra integral model, which characterizes the law of variation between stresses  and strains  in the middle surface, is written in integral form [66]

(1)

where Е – is an elasticity modulus; μ– the Poisson ratio; – integral operator with the kernel of relaxation , 0<<1: ;  t – time of observation;  - time, before the time of observation.

Fig. 1. Geometry of the cylindrical shell.

Geometric relationship between the strains  in the middle surface and the displacements u, v, w in thex, y, z directions for the shells is represented in a nonlinear form [65]

(2)

where кх, ку– are the curvature parameters.

At bending in the middle surface (Fig. 2), there are the forces:

(3)

where h – is a shell thickness.

Moments ,  andare determined through the deflection function w:

(4)

………………………………………………………………………………………………………………………………………………………..

Disregarding the wave character of strain propagation and neglecting the inertial forces in u and v directions (along x axis and tangential to the shell , see Figure 1) and nonlinear terms, from the system of equations (8), we obtain integro-differential equations of motion for viscoelastic cylindrical shell:

(13)

- where  Ф is the stress function.

3. Discrete model

The solution of the systems of nonlinear IDE in partial derivatives, obtained in the previous section under different boundary conditions and in the presence of weakly singular heredity kernels, represents significant mathematical difficulties. Therefore, the natural way to solve these systems is to discretize with respect to spatial variables, and to obtain a system of ordinary IDE with respect to time functions. These equations will henceforth be called the basic resolving IDE of the problems of dynamics of viscoelastic systems. It is shown that when constructing a discrete model of nonlinear problems in the dynamics of the hereditary theory of viscoelasticity, the most effective method is the Bubnov-Galerkin variation method. The advantages of this method are:

- it allows one to solve complex problems for any boundary conditions with known coordinate functions;

- it is universal, as it suits for both linear and nonlinear problems of the hereditary theory of viscoelasticity;

- it ensures the convergence of the solution in the form of a functional series.

In this paper, the main resolving IDEs areis obtained by the Bubnov-Galerkin method.

With the assumption from [65] and assuming that  y = Rθ  (Fig. 1), the equation of oscillations of the viscoelastic cylindrical shell (8) with geometric nonlinearity has the form:

(14)

The problem is solved under the following boundary conditions:

at                                      (15)

An approximate solution of system (14) is sought in the form:

(16)

……………………………………………………………………………………………………………………..

Here-is a velocity of sound,  - dimensionless coefficients.

4. Methods of solution

Further, a numerical method is applied to the system (17), which describes nonlinear problems of oscillations of viscoelastic pipelines [68, 69]. This method is based on the use of various analytic transformations that make it possible to reduce the initial systems to the systems of integral equations with regular kernels and stable numerical integration ensuring the solution of problems with a high degree of accuracy. Since the integral in system (17) has a weak Abel-type singularity, it is impossible to use it directly to calculate a quadrature formula. Therefore, by changing the variables

,    (0<α<1)                                                          (18)

integral with the Koltunov-Rzhanitsyn kernel with a singularity of the following form

А                                                (19)

………………………………………………………………………………………………………………

Thus, according to a numerical method with respect to unknowns, a system of algebraic equations is obtained. The Gauss method is used to solve the system. Based on the developed algorithm, a package of applied computer programs is created.

4.1. Solution of the test cases

Verification of efficiency of the proposed numerical method and programs, based on the solution of test cases, is a necessary stage to confirm the reliability of research results obtained in solving specific problems. The problems for which an exact solution is known [68] have been considered as test cases. Tables 1 and 2 show a satisfactory agreement of approximate solutions with exact ones; this shows the reliability and high accuracy of calculation results.

- Test Example 1. Consider a non-linear integro-differential equation of the form

(24)

w(0)=1, ,

where

, 0<<1;

………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Table 1

Comparison of exact and approximate solutions of the IDE

| t | Solution |  |  |
| --- | --- | --- | --- |
|  | Exact | Approximate |  |
| 0 | 1,000000 | 1,000000 | - |
| 1 | 0,970445 | 0,970373 | 0,710-4 |
| 2 | 0,941764 | 0,941622 | 1,410-4 |
| 3 | 0,913931 | 0,913644 | 2,810-4 |
| 4 | 0,886920 | 0,886569 | 3,510-4 |
| 5 | 0,860707 | 0,860271 | 4,310-4 |
| 6 | 0,835270 | 0,834855 | 4,110-4 |
| 7 | 0,810584 | 0,810278 | 310-4 |
| 8 | 0,786627 | 0,786113 | 5,110-4 |
| 9 | 0,763379 | 0,763126 | 2,510-4 |
| 10 | 0,740818 | 0,740509 | 310-4 |

………………………………………………………………………………………………………………………….

From the table it follows that the error Δh  of calculations performed by described method coincides with the error of the quadrature formulas used and has the same order of smallness relative to the interpolation step (for the trapezoid formula the error of the method with respect to the interpolation step is of second-order, for the Simpson formula – of third order, etc.).

5. Numerical results

5.1. The effect of flow velocity on natural frequencies of pipeline oscillations

Let us investigate the effect of fluid flow velocity on the natural oscillations frequencies of pipes of cylindrical shell type at given boundary conditions (15).

The solution of simplified system (13) by the Bubnov-Galerkin method is sought in the form

(29)

where  wn,m=wn,m(t)   are the sought for time functions. Substituting (29) into the second equation in (13), stress functions in the middle surface are found.

………………………………………………………………………………………………………………………………………………….

Fig. 11. Deflection – time dependence at =1,1(1), =1,5(2), =5(3);

А=0,05; =0,25; =0,005; =0,02;  E=2∙106;  N=2; M=5; М1=0,1.

Fig. 11 shows the curves of deflection w – time dependence of the viscoelastic pipeline at different values ​​of the parameter =R/h. The calculation was carried out for viscoelastic pipes with a relative thickness =R/h, varying in the range from 1.1 to 5. Analysis of these curves suggests that a decrease in the thickness of the pipeline leads to an increase in the frequency of oscillations. In Fig. 11  there is a noticeable increase in the deflection amplitude of the pipeline at =5  (curve 3).

The effect of the Mach number М1 on the behavior of viscoelastic pipelines is studied (Fig. 12). It is seen that with an increase in М1, the amplitude increases proportionally. The greater the value of М1, the greater the amplitude of the oscillations and vice versa.

Fig. 13 shows the time variation of the displacement w of the midpoint of viscoelastic pipelines of cylindrical shell type, obtained from various theories: the linear theory (curve 1) and the nonlinear theory (curve 2).

According to Fig. 13, the results of linear and nonlinear theories differ significantly from each other. Although the solutions of the problems of linear and nonlinear theories differ little in the initial period of time, in the course of time the geometric nonlinearity exerts a significant influence on the solution.

5.3. Determination of critical fluid velocity

6. Conclusions

It should be noted that the algorithm of the proposed method makes it possible to investigate in detail the influence of geometric nonlinearities and viscoelastic properties of structure material on the vibrational processes of viscoelastic pipelines, in particular, in the study of free oscillations of pipelines based on the theory of ideally elastic shells.

In the modeling of linear and nonlinear problems, a number of new dynamic effects have been studied:

- it is established that an account of viscoelastic properties of pipeline material leads decrease in the amplitude and frequency of oscillations;

- it is revealed that an increase in the pipeline thickness (with a decrease in δ parameter) leads to a decrease in the amplitude of oscillations;

- it is shown that an increase in geometric parameter γ and the Mach number М1 leads to an increase in the amplitude and frequency of oscillations;

- it is established that an account of nonlinear effects in solving problems of nonlinear oscillations of viscoelastic pipelines leads decrease in the amplitude and frequency of oscillations.

The results of numerical simulation can be used in oil and gas industry, and in design organizations.

References

- A.N. Anoshkin, V.Yu. Zuyko, S.G. Ivanov, Calculation of Stress-strain State and Prediction of the Strength of Polymer Reinforced Gas Pipes, Bulletin of the Samara State University. Natural science series. 6 (2007) 419-426.

- E.Z.Yagubov, N.D.Tskhadaya, Z.Kh.Yakubov, Multichannel Pipelines for of Oil and Gas Transportation and Recovery of Worn out Oil and Gas Pipelines, Scientific papers. 1 (2013) 57-63.

- L.F. Kochneva, Internal Friction in Solids under Oscillations, Nauka, Moscow, 1979.

- V.D. Potapov, Investigation of the Dynamic Strength of Viscoelastic Systems Using Lyapunov Exponents, Proceedings of the Academy of Sciences. Mechanics of a solid body. 6 (2000) 82-90.

- V.I. Matyash, On the Dynamic Strength of a Hinged Supported Elastic-Viscous Rod, Mechanics of Polymers. 2 (1971) 293-300.

- K. Urbanowicz, M. Firkowski,  Z. Zarzycki, Modelling Water Hammer in Viscoelastic Pipelines: Short brief, Journal of Physics: Conference Series.  760 (2016) 012037.

- L.H. Taïeb, L. Ayed, E.H. Taïeb, Leak Detection in Viscoelastic Pipe by Transient Analysis, in: T. Fakhfakh, W. Bartelmus, F. Chaari, R. Zimroz, M. Haddar (Eds.), Condition Monitoring of Machinery in Non-Stationary Operations, Springer, Heidelberg, Berlin, 2012, pp. 69-79.

- Ghiyam Eslami, Vahid A. Maleki, Mousa Rezaee, Effect of Open Crack on Vibration Behavior of a Fluid-Conveying Pipe Embedded in a Visco-Elastic Medium, Latin American Journal of Solids and Structures. 13 (2016) 136-154.

- Petr A.Velmisov,  Andrei V. Korneev, Mathematical Modeling in the Problem of Dynamic Stability of a Pipeline, Automation of control Processes. Journal of science. 1 (2015) 74-81.

- A. Bergant, Q. Hou, A. Keramat, A.S. Tijsseling, Waterhammer tests in a Long PVC Pipeline with Short Steel End Sections, Scientific professional Quarterly Spring.  1 (2013) 23-34.

- M. Faraji Oskouie, R. Ansari, F. Sadeghi, Nonlinear vibration analysis of fractional viscoelastic Euler–Bernoulli nanobeams based on the surface stress theory, Acta Mechanica Solida Sinica. 30 (2017) 416-424.

- M. Faraji Oskouie, R.Ansari, Linear and nonlinear vibrations of fractional viscoelastic Timoshenko nanobeams considering surface energy effects, Applied Mathematical Modelling. 43 (2017) 337-350.

- X.Q. Zhou, D.Y. Yu, X.Y. Shao, C.Y. Zhang, S. Wang, Dynamics Characteristic of Steady Fluid Conveying in the Periodical Partially Viscoelastic Composite Pipeline, Composites. Part B: Engineering. 111 (2017) 387-408.

- Lazhar Ayed, Lamjed Hadj Taïeb, Ezzeddine Hadj Taïeb,Two Leaks Detection in Viscoelastic Pipeline Systems by Means of Transient Analysis, Journal of Loss Prevention in the Process Industries. 26 (2013) 1341-1351.

- R. Zanganeh, A. Ahmadi, A. Keramat, Fluid-structure Interaction with Viscoelastic Supports during Water hammer in a Pipeline, Journal of Fluids and Structures. 54 (2015) 215–234.

- K.Weinarowska – Bords, Viscoelastic Model of Water Hammer in Single Pipeline – Problems and Questions Arch. Hydro-Eng. Environ. Mech.  53 (2006) 331–351.

- A. Soares, D. Covas, L.F. Reis, Analysis of PVS Pipe-Wall Viscoelasticity during Water Hammer, J. Hydraul. Eng. 134 (2008) 1389–1394.

- Dídia Covas, Ivan Stoianov, João F. Mano, Helena Ramos, Nigel Graham, CedoMaksimovic, The Dynamic Effect of Pipe-wall Viscoelasticity in Hydraulic Transients. Part I—experimental analysis and creep characterization, Journal of Hydraulic Research. 42 (2004) 516–530.

- Petr A.Velmisov, Andrey V.Ankilov, Mathematical Modelling of Dynamics and Stability of Elastic Elements of Vibration Devices, IFAC-Papers On Line. 48 (2015) 970-975.

- M.H. Ghayesh, H. Farokhi, A. Farajpour, Chaotic oscillations of viscoelastic microtubes conveying pulsatile fluid, Microfluidics and Nanofluidics.  22 (2018) 72.

https://DOI: 10.1007/s10404-018-2091-z.

- Ye Tang, Yaxin Zhen, Bo Fang, Nonlinear vibration analysis of a fractional dynamic model for the viscoelastic pipe conveying fluid, Applied Mathematical Modelling. 56 (2018) 123-136.

- R. A. Ibrahim, Overview of Mechanics of Pipes Conveying Fluids-Part I:Fundamental Studies, Journal of Pressure Vessel Technology. 132 (2010).

https://doi: 10.1115/1.4001271.

- Francesco Tornabene, Alessandro Marzani, Erasmo Viola, Isaac Elishakoff, Critical Flow Speeds of Pipes Conveying Fluid Using the Generalized Differential Quadrature Method, Adv. Theor. Appl. Mech. 3 (2010) 121-138.

- A. Setoodeh, S. Afrahim, Nonlinear dynamic analysis of FG micro-pipes conveying fluid based on strain gradient theory, Composite Structures, 116 (2014):128–135.

- J. Tian, C. Yuan, L. Yang, C. Wu, G. Liu, and Z. Yang, The Vibration Characteristics Analysis of Pipeline under the Action of Gas Pressure Pulsation Coupling, Journal of Failure Analysis and Prevention. 16 (2016) 499–505.

- A.M. Dehrouyeh-Semnani, M. Nikkhah-Bahrami, M.R.H. Yazdi, On nonlinear vibrations of micropipes conveying fluid. Int. J. Eng. Sci. 117 (2017) 20–33.

- J. Gu, T. Ma, M. Duan, Effect of Aspect Ratio on the Dynamic Response of a Fluid-conveying Pipe using the Timoshenko Beam Model, Ocean Engineering. 114 (2016) 185–191.

- R. Gomes da Rocha, F. Bastos de Freitas Rachid, Numerical Solution of Fluid—Structure Interaction in Piping Systems by Glimm'sMethod,Journal of Fluids and Structures. 28 (2012) 392–415.

- M. Chen, G.-W. Jiao, S.-S. Deng, J.-H. Wang, Flux Vector Splitting Solutions for Coupling Hydraulic Transient of Gas-liquid-solid Three-phase Flow in Pipelines, Applied Mathematics and Mechanics. 34 (2013) 811–822.

- Ming Chen, Jimiao Duan, Dan Shu, Long Huang, Xiaochen Chen, Study on Influence of Fluid Parameters on Axial Coupled Vibration of Pipeline Conveying Multiphase Flow, International Journal of Chemical Engineering. 2017 (2017).

https://doi.org/10.1155/2017/4824376.

- Usik Lee,  Joohong Kim, Dynamics of Branched Pipeline Systems Conveying Internal Unsteady Flow,Journal of Vibration and Acoustics. 121 (1999) 114-122. https://doi:10.1115/1.2893937.

- Dianlong Yu, Michael P. Païdoussis, Huijie Shen, Lin Wang, Dynamic Stability of Periodic Pipes Conveying Fluid, J. Appl. Mech.  81 (2014) 011008-1-011008-8. https:// doi: 10.1115/1.4024409.

- Hiroaki Furuya, Kiyotaka Yamashita, Hiroshi Yabuno, Nonlinear Stability of a Fluid-Conveying Cantilevered Pipe With End Mass in Case of Horizontal Excitation at the Upper End. ASME 2010 7th International Symposium on Fluid-Structure Interactions, Flow-Sound Interactions, and Flow-Induced Vibration and Noise: Volume 3, Parts A and B. Montreal, Quebec, Canada, August 1–5, 2010. FEDSM-ICNMM2010-31239. (2010) 1219-1227.

- M. Stangl, J. Gerstmayr, H. Irschik, A Large Deformation Planar Finite Element for Pipes Conveying Fluid Based on the Absolute Nodal Coordinate Formulation, ASME Journal of Computational and Nonlinear Dynamics, 4 (2009) 031009-1 - 031009-8.

https://doi:10.1115/1.3124091.

- E. B. Wylie, Frictional Effects in Unsteady Turbulent Pipe Flows. Appl. Mech. Rev. 50 (1997) 241-244.

- Motohiko Nohmi, Satoshi Yamazaki, Shusaku Kagawa, Byungjin An, Donghyuk Kang, Kazuhiko Yokota, Numerical Study of One Dimensional Pipe Flow Under Pump Cavitation Surge. FEDSM2017-69427, pp. V01AT05A023; ASME 2017 Fluids Engineering Division Summer Meeting (2017).

- Athanasia Kalpakli, Vester Ramis Örlü, P. Henrik Alfredsson, Turbulent Flows in Curved Pipes: Recent Advances in Experiments and Simulations. Appl. Mech. Rev. 68 (2016).

- L.D. Akulenko, M.I. Ivanov, L.I. Korovina, S.V. Nesterov, Basic Properties of Natural Oscillations of an Extended Section of a Pipeline, Mechanics of a solid body.  4 (2013) 119-134.

- M.P. Païdoussis, The Canonical Problem of the Fluid-conveying Pipe and Radiation of the Knowledge Gained to other Dynamics Problems inApplied Mechanics, J. Sound and Vibr. 310 (2008) 462–492.

- M. Wadham-Gagnon, Michael P. Païdoussis, C. Semler, Dynamics of Cantilevered Pipes Conveying Fluid. Part I: Nonlinear equations of three-dimensional motion, Journal of Fluids and Structures. 23 (2007) 545-567.

- O. Tkachenko, Construction of Mathematical Model of Complex Pipeline with Variable Geometry. 15th International scientific conference “Underground urbanization as a Prerequisite for Sustainable Development”, Proc. Engineering. 165 (2016) 1261-1274.

- Mohamed F.Khalil, Sadek Z. Kassab, Ihab G. Adam, Mohamed A. Samaha,  Turbulent Flow around Single Concentric Long Capsule in Pipe,  Applied Mathematical Modelling.  34 (2010) 2000-2017.

- M.R. Ghazavi, H. Molki, A. Ali beigloo, Nonlinear analysis of the micro/nanotube conveying fluid based on second strain gradient theory, Applied Mathematical Modelling. 60 (2018) 77-93.

- Yong Guo, Jianhua Xie, Lin Wang, Three-dimensional vibration of cantilevered fluid-conveying micropipes—Types of periodic motions and small-scale effect, International Journal of Non-Linear Mechanics. 102 (2018) 112-135.

- Ye-Wei Zhang, L. Zhou, B. Fang, Tian-Zhi Yang, Quantum effects on thermal vibration of single-walled carbon nanotubes conveying fluid, Acta Mechanica Solida Sinica. 30 (2017) 550-556. https://doi.org/10.1016/j.camss.2017.07.007

- L. Wang, Y. Hong, H. Dai, Q. Ni, Natural frequency and stability tuning of cantilevered CNTs conveying fluid in magnetic field, Acta Mechanica Solida Sinica. 29 (2016) 567-576.

- Z. Zhang, Y. Liu, H. Zhao, W. Liu, Acoustic nanowave absorption through clustered carbon nanotubes conveying fluid,  Acta Mechanica Solida Sinica. 29 (2016) 257-270.

- L. Wang, H. Hu, Thermal vibration of single-walled carbon nanotubes with quantum effects, Proc. R. Soc. A 470 (2014) 20140087.

- Xiao-wen, Zhou Hu-Liang Dai, Lin Wang, Dynamics of axially functionally graded cantilevered pipes conveying fluid, Composite Structures, 190 (2018) 112-118.

- L. Wang, Z.Y. Liu, A. Abdelkefi, Y.K. Wang, H.L. Dai,  Nonlinear dynamics of cantilevered pipes conveying fluid: Towards a further understanding of the effect of loose constraints, International Journal of Non-Linear Mechanics. 95 (2017) 19-29.

- G.L. Kuipera, A.V. Metrikine, Experimental investigation of dynamic stability of a cantilever pipe aspirating fluid, Journal of Fluids and Structures. 24 (2008) 541–558.

- J. Singh, M. Rudman, H.M. Blackburn, The influence of shear-dependent rheology onturbulent pipe flow, J. Fluid Mech. 822 (2017) 848-879.

- L. Wang, Q. Ni, Vibration of Slender Structures Subjected to Axial Flow or Axially Towed in Quiescent Fluid, Advances in Acoustics and Vibration. (2009), Article ID 432340.

- Jeng-Shian Chang, Wen-Jiann Chiou, Natural frequencies and critical velocities of fixed-fixed laminated circular cylindrical shells conveying fluids, Computers & Structures. 57 (1995) 929-939.

- G.F. Safina, Preservation of the frequencies of oscillations of a pipe with a liquid, Sibirskii  Zhurnal  Industrial'noi  Matematiki. 15 (2012) 124-134.

- V.A. Rukavishnikov, O.P. Tkachenko, Approximate Solution to the Nonlinear Problem of an Underground Pipeline Deformation, Journal of Applied and Industrial Mathematics. 6 (2012) 100–110.

- R. Kolahchi1, A. Ghorbanpour Arani, Nonlinear Vibration and Instability Analysis of a PVDF

Cylindrical Shell Reinforced with BNNTs Conveying Viscose Fluid Using HDQ Method, Journal of Solid Mechanics. 4 (2012) 267-276.

- Jiecheng Yang, Ying Min Low, Cheng-Hsien Lee, Yee-MengChiew, Numerical simulation of scour around a submarine pipeline using computational fluid dynamics and discrete element method, Applied Mathematical Modelling. 55 (2018) 400-416.

- Ioannis K. Chatjigeorgiou, Second-order nonlinear dynamics of catenary pipelines subjected to bi-chromatic excitations, Applied Mathematical Modelling. 39 (2015) 2363-2384.

- Y. Zhang, M. Zhao, K. C. S. Kwok, M. M. Liu, Computational fluid dynamics–discrete element method analysis of the onset of scour around subsea pipelines, Applied Mathematical Modelling. 39 (2015) 7611-7619.

- Alfredo López-Benito, Francisco J. Elorza Tenreiro, Luis C. Gutiérrez-Pérez, Steady-state non-isothermal flow model for natural gas transmission in pipes, Applied Mathematical Modelling. 40 (2016) 10020-10037.

- L. Wang, H. Hu, W. Guo, Thermal vibration of carbon nanotubes predicted by beam models and molecular dynamics. Proc. R. Soc. A. 466 (2010) 2325–2340.

- F. X. Chu, J.J. Li, Liquid-Solid Coupling Vibration Analysis of the Offshore Pipeline, Advanced Materials Research. 926-930 (2014) 869-872.

- A.S. Volmir, Shells in the Flow of Fluid and Gas, Problems of Hydro-elasticity, Science, Moscow, 1979.

- E.I. Grigolyuk, V.I. Mamay, Nonlinear Strain of Thin-Walled Structures, Science, Moscow, 1997.

- M.A. Koltunov, Creep and Relaxation, Higher School, Moscow, 1976.

- V.V. Bolotin, Non-conservative Problems of the Theory of Elastic Strength, Fizmatgiz, Moscow, 1961.

- F.B. Badalov, Methods for Solving Integral and Integro-differential Equations of the Hereditary Theory of Viscoelasticity, Mehnat, Tashkent, 1987.

- F.B. Badalov, Kh. Eshmatov, M. Yusupov, Some Methods of Solution of Systems of Integro-differential Equations Encountered in Problems of Viscoelasticity, Applied Mathematics and Mechanics. 51(1987) 867-871.

- B.A. Khudayarov, Numerical Analysis of the Nonlinear Oscillation of Viscoelastic Plates, International Applied Mechanics. 41 (2005) 538–542.

Corresponding author. Present address: Department of  Higher Mathematics, «Tashkent institute of irrigation and agricultural mechanization engineers» NRU , str. Kari-Niyazova, 39, Tashkent, 100000, Uzbekistan. Tel.: +99871-2370986.

E-mail: bakht-flpo21@yandex.ru,  khudayarovba07@gmail.com (B.A. Khudayarov).